# Sonic Firmware Documentation V49

◀️  Join the SonicFw Telegram https://t.me/SonicFw

📺 Youtube Channel https://www.youtube.com/@robby_69400

💾 DOWNLOAD select RAW file DOWNLOAD_HERE

🙏 Many thanks to Sonic Team : Zylka, Kolyan, Iggy, Toni, Yves, Francois and Adam.

This software is a fork of Armel F4HWN firmware with code from NTOIVOLA, EGZUMER and DUAL TACHYON.

## EXPLANATION OF BUTTON SYMBOLS:

1 – Simple press.

L1 – Long press.

F1– Press after the F button.

## MR MAIN SCREEN (CHANNELS) | VFO (FREQUENCY)

L1 – (MR) – Transfer the channel frequency to VFO mode.

L2 – (MR and VFO) – VFO change (in DUAL icon indication).

L3 – Change the MR/VFO mode on the selected channel.

L4 – (MR and VFO) – Bandwidth change.

L5 – (MR) – Change the list number for the channel. (VFO) – Change of step.

L6 – (MR & VFO) – Power Change (L/M/H).

L7 – (MR and VFO) – On/Off flashlight indication when receiving a signal.

L9 – (MR and VFO) – On/Off constant backlight.

L0 – AM/FM/USB Modulation Change

F2 – (MR) – Delete Channel. (VFO) – Preserve the frequency.

F6 – Spectrum analyzer in scanrange mode.

F7 – Spectrum analyzer in scanlist mode.

F8 – Band Spectrum Analyzer.

F9 – Spectrum analyzer in frequency mode.

F0 – Radio broadcasting (FM radio).

SK1 and SK2—Side keys, assignable buttons.

<= and => – Navigation buttons.

MA – Menu.

EXIT – Exit.

#F – Functional "F".

## OPERATION IN SPECTRUM MODES

Now the scan is interlaced: the step is minimal 25k and the spectrum is swept with different start frequencies to cover all the frequencies.

For example 125.100, 125.125,… then 125.10833,125.133333…then 125.11666,125.3666…

This to improve detection.

Different types of spectrum are called by the quick buttons

F6 - Range, Freq start, stop and step set in [5] menu.

Scans between 2 selected frequencies with the given step

F7 - Scanlists

Scans the frequencies in scanlists that can be defined in VFO or Chirp*.

20 lists available, maximum 500 channels to scan.

F8 - Bands

Scans ranges that are defined in bands, can be defined in Chirp* or in a channel greater than 974

Channel Frequency is START, Offset + is STOP, STEP is step.

F9 - Frequency: spectrum around VFO frequency, 128 frequencies with current step.

*See Chirp section below.

1 – Skip the frequency in the scan mode.

2 – Turn on/off the backlight).

3 – Bandwidth in the spectrum. In the options menu, change the value.

4 – List of scanned lists or bands.

5 – Options menu. Navigation through the menu < and >. Change the values with buttons 1 and 3.

6 – Changing the modes of the spectrum analyzer Scanlists / Bands.

7 - Save Settings. Save after configuring and selecting the lists and the options will be applied once enabled.

8 – Change the display screen between Classic spectrum, Smooth spectrum and SCAN.

Smooth Spectrum	Classic Spectrum

9 – Change of AM/FM/USB modulation.

0 – Switch to the history mode. Displays the RX frequencies, channels, code and time in s.

* and #  DYNSQL Level setting  - RSSI spike threshold  for instant peak capture. For weak and fuzzy signals, increase or disable.

L* and L# DYNSQL OFF This will fill the history, but won’t stop.

SK1 – Frequency Lock (FL) then monitor and back to normal mode.

FL locks the frequency, opens listening on receive

Monitor, keeps the frequency locked and  open listening

SK2 – Blacklist a frequency, it is visible in history with a #

## Keys in history

<,> - Move to previous / next item, automatically set to frequency lock to the selected item

1 – Clears all history (use 8 to save).

2 – Saves the frequency to a free channel.

3 – Delete the entry.

4 – Clears all black listed entries (use 8 to save).

5 – Scans history entries.

6 – Launch close call

7 - Clears all non black listed entries (use 8 to save).

8 –Saves the history.

EXIT : go back to SCAN or SPECTRUM

## Keys in band selection

<,> - Move to previous / next item

1,3 – Select a preset

4 – Activate or deactivate one band.

5 – Activate one band and deactivate any other

7 – Save current preset.

MENU -  activate selected item and start scanning

EXIT : go back to SCAN or SPECTRUM

## Keys in scanlist selection

<,> - Move to previous / next item

4 – Activate or deactivate one scanlist.

5 – Activate one scanlist and deactivate any other

MENU -  activate selected item and start scanning

EXIT : go back to SCAN or SPECTRUM

## PARAMETERS OF THE SPECTRUM ANALYZER

SPECTRUMDELAY – Pause time after signal detection (in ms) – if 0, immediately resumes scanning.

MAXLISTENTIME – Maximum time to listen to the signal (in seconds) – after that, the scan continues.

FSTART – Initial frequency of the scan range (in MHz).

FSTOP – End frequency of the scan range (in MHz).

STEP – Frequency scan step (in kHz): 0.01; 0.05; 0.1; 0.5; 1.0; 2.0; 5.0; 6.25; 10.0; 12.5; 20.0; 25.0; 50.0; 100.0.

LISTENBW – Bandwidth: Wide (25 kHz) / Narrow (12.5 kHz)/ AIR (8.33kHz).

MODULATION – Modulation type: FM/AM/USB.

POWERSAVE – Power saving mode: Off /200ms /500ms /1s /2s /5s.

KEY UNLOCKED – THE TIME BEFORE THE KEYPAD IS LOCKED.

NOIS LVL OFF – Noise level for disabling reception (above this – the signal is counted).

POPUPS – Popup display time (in ms) – 0 = disabled.

RECORD TRIG – An additional threshold for recording in history. The higher the threshold, the stronger the signal will be recorded.

SOUND BOOST – Amplify sound. Increase bass frequencies.

PTT – What to do when pressing PTT: LAST VFO /NINJA /LAST RECEIVED/AUTO ROGER .

- NINJA chooses a random frequency in the current scanlist.

- AUTO ROGER :Choose your Roger in vfo menu, select a frequency or a channel in vfo, in spectrum select ptt auto Roger in the menu, it will send a Roger beep every duration you select.

MONITOR SL – Includes the "MON" priority list in all scan lists.

RESET DEFAULT – Reset all spectrum parameters to factory settings.

## DETAILED MONITORING IN SPECTRUM MODE VIA THE "MENU" BUTTON

Move through the low-level register menu with buttons 2 and 8. Change values with the <= and => buttons

LNAS – Low Noise Amplifier (LNA) – signal amplification stages in front of the mixer.

LNA – Gain of the first LNA stage – higher = stronger signal but more interference.

PGA – Programmable Gain Amplifier (after LNA). – Balance between sensitivity and distortion.

MIX – Mixer Gain (Frequency Mixing) – Higher = Better Dynamic Range but More Noise.

XTAL F MODE SELECT – Quartz filter (XTAL) operating mode. Filtering (reduces noise, but can distort).

OFF AF RX DE-EMP – Disable demphasis (pre-distortion correction) during reception.

GAIN AFTER FM DEMOD – Adjusts the reception volume.

RF TX DEVIATION – Transmission frequency deviation (Fm deviation) – affects the transmission bandwidth (higher = louder).

COMPRESS AF TX RATIO – Compression ratio of audio during transmission – compresses the dynamic range.

COMPRESS AF TX 0 DB – Compression level 0 db during transmission – the compression start point.

COMPRESS AF TX NOISE – Noise reduction in quiet areas.

MIC AGC DISABLE – Disable automatic microphone gain control (AGC).

AFC RANGE SELECT – Operating range of AFC (Automatic Frequency Tuning).

AFC DISABLE – Disable AFC.

AFC SPEED – AFC Speed.

## OPERATION IN BROADCAST RADIO MODE "FM"

The firmware has a simplified FM radio. Use the <= and => buttons  to search for the nearest available broadcasting frequency in automatic mode.  The * button  will change the Auto mode to manual search.

Press key 1 to 6 to select the corresponding memory.

Long press key 1 to 6 to save a frequency to that memory.

Press 0 to toggle receiving the VFO frequency, RX MON displayed or removed.

## MAIN MENU

THERE IS NO HIDDEN MENU IN THE FIRMWARE

|  | Step | The frequency step (in kHz), UP and Down buttons change the frequency to this value, and you can only set the frequency to half of this value. For CB - 5/10 kHz; SatCom, Avia - 5/25 kHz; The rest is 6.25/12.5 kHz. |
| --- | --- | --- |
|  | SQL | The level of noise cancellation (Squelch) is from 0 (always open) to 9 (very strict). Set for both bands (A and B) |
|  | Mode | Demodulation mode, default - FM, AM and USB can only be used for listening |
|  | RxMode | Sets how the high and low frequencies are used on the screen (MAIN ONLY - Always transmits and listens on the main frequency (MO), DUAL RX RESPOND - Listens to both frequencies, if the signal is received on the secondary frequency, it is fixed on it for a couple of seconds so that you can answer the call (DWR), CROSS BAND - always transmits on the primary frequency and listens on the secondary frequency (XB), MAIN TX DUAL RX - always transmits to the main, listens to both (DW). |
|  | RxDCS | Setting Digital Coded Squelch, if enabled, the squelch will only open after receiving that subtone, applicable for FM modulation only. |
|  | RxCTCS | Setting an analog subtone to reception (Continuous Tone Coded Squelch), if the function is enabled, the noise canceler will open only after receiving this subtone, applicable only for FM modulation. |
|  | TxDCS | Setting the digital encoded digital subtone to transmit, if the function is enabled, the transceiver will transmit this tone, applicable only for FM modulation. |
|  | TxCTCS | Setting the analog subtone to transmit, if the function is enabled, the transceiver will transmit this tone, applicable for FM modulation only. |
|  | TxOffs | Transmitter frequency offset value in MHz, transmit/receive at different frequencies. |
|  | TxODir | Direction (+/-) of transmitter frequency offset when transmitting/receiving at different frequencies |
|  | W/N | The bandwidth used by the transceiver (WIDE (25 kHz), NARROW (12.5 kHz). |
|  | SetNFM | Sets Narrow FM to Narrow or Narrower |
|  | Scramb | Selects Scrambling type |
|  | Scraen | Enables Scrambler |
|  | Compnd | Compander - Soft audio filter for clearer sound. Amplifies the received desired signal even when the signal is weak at the transmitting microphone, improving the signal-to-noise ratio. To work, it must be turned on on both walkie-talkies. |
|  | Roger | On/off "roger beep" (end-of-pass tone). |
|  | STE | Noise suppressor, eliminates noise at the end of the transmission. When PTT is released, a short 55Hz tone is emitted, signaling to other transceivers to mute |
|  | RP STE | Noise Cancelling Tail Eliminator Repeater |
|  | ChSave | Saves the frequency to a chosen memory |
|  | ChDele | Deletes a channel memory |
|  | ChName | Name of the memory channel. |
|  | ChDisp | Choose how the saved channel will be displayed on the walkie-talkie display |
|  | F1Shrt | Function assignment by short pressing the F1 side key. |
|  | F1Long | Function assignment when the F1 side key is pressed for a long time. |
|  | F2Shrt | Function assignment by short pressing the F2 side key. |
|  | F2Long | Assign a function when you press the F2 side key for a long time. |
|  | M Long | Assign a function when you long press the M key |
|  | KeyLck | Automatically lock the keypad after a set time |
|  | TxOut | Transmit Timeout - The amount of time that PTT can be held during transmission (in seconds). Protection against accidental clamping of PTT. |
|  | BatSav | Battery Save: OFF / 1:1 / 1:2 / 1:4 / 1:8. |
|  | Mic | Microphone sensitivity (microphone amplifier). |
|  | POnMsg | Enable/disable the display of a message when turned on (Power On Message). |
|  | BLTime | Screen backlight time when buttons are pressed. |
|  | BLMin | Minimum backlight brightness (in percentage). |
|  | BLMax | Maximum backlight brightness (in percentage). |
|  | BLTxRx | Transmit/Receive Backlight: OFF/TX/RX/TX+RX. |
|  | SetInv | Enable walkie-talkie screen color inversion |
|  | BtnInv | Reverse the direction of the UP and DOWN buttons |
|  | SetLck | Setting the keypad lock type with or without PTT. |
|  | BatTxt | Battery Display Type, %, V or disabled. |
|  | MicBar | Enabling/disabling the microphone bar |
|  | SetTmr | Set the display of RX/TX timers on the screen |
|  | F Lock | Setting up Frequency Lock. What frequencies are allowed or prohibited to work on. |
|  | BatCal | Battery Calibration. |
|  | BatTyp | Choice of battery type (Li-ion, NiMH, etc.) and capacity. |
|  | Reset | Channel Preservation Reset (NOCH) or Full Factory Reset including Saved Channels (ALL) |

# Installation

RS232 version uses the kenwood cable and has 200 history entries

USB version uses USB-C cable and has 50 history entries because the USB driver takes more RAM memory.

RS232 Kenwood cable	USB-C cable

use the same procedure as F4HWN, after that :

- If necessary, you can do a full reset of the radio. press the PTT and Fn2 buttons  , turn on the radio. A warning message will be displayed on the screen. hold the buttons for 4 seconds before resetting.

In case of issues to install, try to first install N7SIX firmware to reset the memory.

## CHIRP Driver

Chirp_Sonic.py in DOWNLOAD_HERE

This driver also allows

- customization of the 50 bands on channels 976 to 1024.

Select start frequency  and offset as stop frequency, step and modulation

## Chirp Configuration

- Memory channels 975 to 1024 store custom band settings. For each, define:

  - Band Name.

  - Start and End frequency (via offset).

  - Modulation & Frequency Step.

- Scanlist Monitor setting Select up to 20ch to Monitor scanlist anywhere in the channels.